Press Backgrounder

Media Contact: Grace Capwell

Phone: 1 512 834 7214

Grace.Capwell@Emerson.com

How Advanced Automation Ensures Integrity Across the Carbon Capture Value Chain

As the global economy grapples with climate change and transitions away from fossil energy, breakthroughs in automation are making it possible for carbon capture technology to move from ambition to action by safeguarding the integrity of the process, both physically and fiscally, from the capture point to the storage site and everywhere in between.

Table of Contents

IntroductionPage 2.

Approaching an Inflection Point

Headwinds and Tailwinds

Linking Up the Value ChainPage 4.

Capture and Purification

Compression/Liquefaction

Transportation

Geologic Storage

Cost Integrity: Capture & PurificationPage 7.

Streamlining engineering & process design

Increasing efficiency of heat transfer

Balancing capture rate vs. energy use

Equipment Integrity: Compression/LiquefactionPage 12.

Improving equipment availability

Conducting preventive maintenance

Controlling corrosion

Product Integrity: TransportationPage 14.

Managing purity, density & containment

Facilitating custody transfer

Performance Integrity: Injection & Geologic StoragePage 17.

Assessing geologic characterization

Monitoring and controlling injection

Conclusion: From ‘Boondoggle’ to Net-Zero Imperativepage 19.

Introduction

After decades of promise, carbon capture and storage (CCS) technology is today on the threshold of realizing its potential as a strategic alternative in the global effort to achieve a net-zero emissions economy. Capturing carbon dioxide (CO2) from industrial exhaust or directly from the atmosphere, then compressing, transporting, and storing it permanently in geologic formations deep underground is a complex process involving technical challenges and market forces that have, until recently, held it back as a viable solution to the climate crisis. The deployment of carbon capture in industrial processes dates to the 1930s, when natural gas producers first used carbon dioxide absorption methods with chemical solvents to separate CO2 from methane. For the next eight decades, unjustifiable cost, real and perceived risk, and different public priorities effectively handicapped the development of new and better capture technologies. This was especially true for lower-concentration gas streams such as power plant exhaust, despite the theoretical potential of eliminating greenhouse gas at the source. As a result, funding and research came mostly in fits and starts.

Since the late-2010s, however, with climate change increasingly front and center in the public mind, a significant shift has begun, thanks in large part to a combination of technological innovation aided by economic incentives and new pro-environmental norms taking root in societies around the world. Government and private sector leaders face not just an environmental imperative to remove the last two centuries’ worth of carbon released into the atmosphere from human activity, but also growing economic pressure to achieve strategic energy security while driving down the cost per ton of CO2 removed. The global transition to fully renewable sources will be gradual and not without uncertainty, since some level of reliance on fossil fuels in the near-term is unavoidable, recent analysis points to a wide range of probable scenarios in which CCS grows into one of the most efficient and robust solution in terms of capacity. While lower-carbon streams that use capture technology in conjunction with hydrogen production are already driving strong demand for capacity, the value and effectiveness of CCS is set to take off.

Approaching an Inflection Point

Most leading experts project that the CCS space will see tremendous growth over the next 40 to 50 years. According to the International Energy Agency (IEA) net-zero scenario, the amount of CO2 captured annually is projected to skyrocket by 17,200%, from 40 million metric tons in 2020 to around 6.9 billion metric tons by 2070, with CCS contributing nearly 15% of all emissions removed, second only to electrification [See Fig. 1]. As of 2020 there were at least 26 commercial-scale carbon capture projects operating worldwide. Project developers have announced ambitions for about 200 new facilities to come online by 2030, capturing over 220 million tons of CO2 per year. Only about 10 of those projects under development had reached a final investment decision as of mid-2022, so it’s early days, and there is still more to be proven in terms of economics, logistics, governance, and risk. That said, from a scientific standpoint, it has become all but impossible to imagine meeting future global net-zero goals without CCS.

Governments, companies, trade associations, unions, environmental NGOs, civil society organizations, and citizens are still working to create the right standards, policy frameworks and regulatory structures needed to expand CCS capacity. Meanwhile, the 2015 Paris Agreement’s goal of limiting global warming to 1.5 degrees Celsius (2.7 degrees Fahrenheit) above the benchmark temperature set before the Industrial Revolution is driving stronger climate policies worldwide, including the 2022 U.S. Inflation Reduction Act, which contained an estimated $369 billion in climate funding, and the European Union’s (EU) Green New Deal. In response, companies like Dow, ExxonMobil, BP, and Conoco Phillips are investing heavily in CCS projects and plan to spend billions more over the next five to 10 years.

The trend isn’t limited to oil and gas and chemicals, however. Power companies and large manufacturers, particularly those in difficult-to-abate sectors like cement and steel, are investing in CCS and partnering with startups, which in turn promote innovation and create novel business opportunities. Improving the availability and cost of finance has a significant impact on capture costs, which in turn affects capacity growth. Stakeholders are beginning to pursue this strategy by lowering interest rates through low-cost financing, loan guarantees, master limited partnerships, and private activity bonds. Meanwhile philanthropic initiatives like the $100 million XPRIZE for carbon removal are partnering with technology suppliers to foster breakthroughs, share knowledge, raise awareness, and ultimately drive down CCS technology costs.

Headwinds and Tailwinds

A global-scale CCS industry might seem inevitable, growth projections and urgency around climate change being what they are, but there are still several key challenges that experts and business leaders acknowledge they must address to accelerate development and make sure the industry is sustainable. First among these is lowering the operational costs involved, especially from energy, maintenance, and regulatory compliance. Maintaining CO2 purity while reducing energy use, safely containing fluids under pressure during transport, monitoring the storage site and injection process, and measuring precise quantities of CO2 during custody transfer are all essential tasks for supporting market mechanics and ensuring regulatory compliance. Health and safety are always a concern, particularly due to the high pressures involved throughout the process, and the environmental concerns of storing millions of tons of concentrated greenhouse gas for an indefinite period.

With its power to improve efficiency, reliability, and safety, advanced automation technology is playing a vital role in solving these problems. Digital automation technologies can aid engineering, optimize the capture process, reduce downtime, save energy, control corrosion, and improve the reliability of essential equipment like compressors, heat exchangers, fans, valves, and pumps. Along transport routes and at any point where it changes hands, automation helps to keep CO2 contaminant-free, pressurized, at the ideal density, and fully accounted for in fiscal transactions. It can also analyze the structural integrity of the storage site, monitor displacement in the geologic formation, and continuously manage downhole CO2 behavior to remain within safe limits.

In short, automation makes it easier for companies to move from ambition to action by ensuring the integrity of the process, both fiscally and physically, from the capture point to the storage site and everywhere in between. This backgrounder outlines four major segments of the CCS value chain – capture and purification, compression/liquefaction, transportation, and injection and geologic storage, also referred to as sequestration – and identifies the major business objectives for operators in each [See Fig. 2]. It defines the operational capabilities that enable the enterprise-level goals, then highlights several key competencies that are necessary to achieve each capability. Finally, for the most critical of these, the role of software and automation technology is explained along with the measurable effect it can have on performance, efficiency, reliability, and safety.

Linking Up the Value Chain

While each of the four segments of the CCS value chain involves many unique considerations, constraints, priorities, and risks, they are also all highly interdependent and can set off a cascade of negative effects if even one experiences disruption for any amount of time. And as with all efforts to decarbonize heavy industry, intensive collaboration among public and private sector stakeholders across the entire chain is necessary if capacity is to meet the market’s growth projections and significantly impact climate change.

Capture and Purification –

The first step involves capturing carbon dioxide from industrial exhaust or directly from the atmosphere using different methods. All of the most common methods in use today use a pre-treatment unit typically operating upstream from the capture system to wash the source gas with chilled water to knock down heavy particulates and impurities.

Pre-combustion capture involves converting fossil fuels, such as coal or natural gas, into a gas mixture of hydrogen, carbon monoxide and carbon dioxide, which is separated into its individual components. Carbon monoxide reacts to produce carbon dioxide, which is then captured, while the hydrogen released from the reaction can be used as a fuel. Post-combustion capture methods collect CO2 after an industrial process or power plant produces it as exhaust or a chemical byproduct. Highly specialized chemicals treat the incoming flue gas, causing it to bond with the carbon dioxide molecules, which then separate out and pipe off to the compression unit prior to transport for long-term geologic storage. Meanwhile the chemical filtration media in the stripper unit meanwhile is heated and reused to purify more flue gas [See Fig. 3].

While both methods can be effective in reducing carbon emissions, pre-combustion capture is typically more efficient because it captures CO2 before it is released into the atmosphere. It involves additional processing steps that could require new facilities and be more expensive to implement, particularly since it isn’t easy to update existing processes. Post-combustion capture, on the other hand, can be retrofitted to suit existing industrial processes and power plants, although it is typically less efficient and may require more energy to operate.

Post-combustion capture systems use either absorption, adsorption, or membrane-type separation, each of which have certain advantages and disadvantages based on various factors, like the characteristics and volume of the flue gas containing CO2 and the economic and environmental considerations of the CCS project. Despite its relative cost, absorption is the most common and well understood capture approach. Adsorption can be highly effective but is less prevalent, as it is still in early commercialization stages. Both approaches use considerable amounts of energy to strip captured CO2 from filtration media and chemically regenerate the liquid solvent or solid adsorbent, as explained below. Membrane separation uses a special thin semi-permeable membrane to screen CO2 from other gases. While potentially much less energy-intensive, membrane technology is also in the initial stages of development.

Direct-air capture involves drawing in extremely high volumes of air at low pressure using fan arrays – sometimes the size of football fields – then compressing it prior to the chemical filtration process, since CO2 pulled directly from the atmosphere is much less concentrated than flue exhaust from a power plant or ethylene cracker.

This backgrounder focuses on post-combustion since it is the most developed, widely used, and easiest to implement into existing industrial processes of all currently available capture methods. Direct air capture, which uses large fan arrays and membrane technology to filter carbon dioxide from air, could also eventually account for a large share of future capacity as well, although it is currently more costly due in part to the lower concentration of CO2 in the atmosphere. While direct air capture is likely to play a critical role in the ability of CCS to address long-term global decarbonization goals, post-combustion has the highest potential to address climate change as things stand today, and so will be the focus of discussion here.

Compression/Liquefaction –

After capturing and producing a rich-CO2 stream, equipment compresses the gas to high pressure in preparation for the next step in the value chain. Multiple stages of compressors and intercoolers condense the hot, captured CO2 from near-ambient pressure to pipeline specification, which is typically around 1,100 to 2,200 pounds per square inch (psi) and 60 to 90 degrees Fahrenheit (15 to 32 degrees Celsius). During this process [See Fig. 4], the CO2 is often purposely forced into a supercritical state, which is achieved by increasing the pressure and temperature to a level where the molecules exhibit both gas- and liquid-like properties. Under supercritical conditions, the CO2 has a much higher density and better solubility, reducing the volume that must be shipped and in turn cutting transportation costs.

Managing supercritical or high-density fluids involves additional complexities, some of which are not unique to CO2. Liquifying natural gas, for example, entails several of the same operational requirements, such as ensuring the reliability of rotating equipment and analyzing the volume and purity of the fluid at critical points along the process. The two most common types of compressors used in the CCUS process are high-pressure reciprocating compressors, which work by compressing the CO2 using a piston that moves back and forth in a cylinder, and centrifugal compressors that use a high-speed impeller to accelerate gas to a high velocity and convert its kinetic energy into pressure energy. In addition, heat exchangers remove the thermal energy generated during compression, while coolers lower the temperature of the compressed CO2 itself, and filters remove any impurities or moisture present along the way.

Transportation –

Compressed CO2 is moveable in gaseous, liquid, or supercritical states using a variety of modes, depending primarily on the distance between the capture facility and the storage site. Specialized assets, such as high-strength vessels, tanks, and pipelines, are required to safely transport CO2 at the proper density [See Fig. 5]. The pipelines used for transportation are built to withstand extremely high pressures and are equipped with pumps, compressors, and valves to control flow. LNG carrier ships can bunker large quantities of CO2 in liquid form. Cryogenic pipelines are also sometimes used to keep the fluid in a constant supercritical state. For shorter distances, moving high-density gas or liquid through fossil fuel-grade pipelines are often the most economical and practical.

In addition to keeping the CO2 pure, contained, and at ideal pressure along the transport route, producers need to be able to report the exact amounts that leave the capture point; likewise, operators and storage facilities need to know how to invoice suppliers of CO2. Wherever possession changes hands, all parties must also be able to exactly quantify any volume of fluid lost through leaks and other unintended releases. Custody transfer metering involves meeting unique technical challenges and complex regulatory requirements, but as a mechanism for carbon taxes and emissions trading systems—the ‘cash register’ essentially—it is a fundamental link in the chain.

Injection & Geologic Storage –

The final step in the CCUS value chain involves injecting the compressed CO2 into underground geologic formations such as depleted oil and gas reservoirs, saline aquifers, or coal seams for long-term storage [See Fig. 6]. The goal is to store the CO2 under high pressure in these formations indefinitely, preventing it from entering the atmosphere similarly to the way that natural gas exists sealed underground for millions of years before extraction by humans. The sequestration of compressed CO2 requires specialized assets, such as tanks, valves, instrumentation, and geophysical tools. After an injection well is drilled into a geologic formation, or an existing well is recommissioned, instrumentation, such as pressure sensors and temperature sensors, are installed to measure the injection process and ensure that the CO2 remains safely stored underground. Geophysical tools, such as seismic modeling and electromagnetic surveys, characterize the geologic formations, help identify storage candidates, and ultimately choose the best locations for injection, which must be done far in advance in conjunction with planning new projects at capture points and filing permit applications.

The well injection process, which can involve pressures of up to 15,000 psi, can take several years if not decades before the well is filled, requiring constant monitoring to ensure that the CO2 is injected safely and efficiently. Developments in sequestration techniques include the use of enhanced oil recovery (EOR), where the injected CO2 is used to recover additional oil from existing wells. This can help offset the cost of CCUS by generating additional revenue from the sale of the recovered oil. Another technique is the use of carbon mineralization, where the injected CO2 reacts with minerals in the geologic formations to form stable carbonates. This can further strengthen the long-term integrity of storage reservoirs, which in most cases must be certified for at least 100 years of use.

The ‘U’ in the common acronym ‘CCUS’ stands for ‘utilization,’ which is a relatively new but potentially impactful subsegment of the value chain. The development of CO2 for beneficial use technologies varies widely, but in principle they could all add to the array of solutions needed to achieve decarbonization, further spurring investment and lowering costs across the CCS value chain. Some, like enhanced oil recovery and carbon farming are more established, while others, like chemical and fuel production and cement manufacturing are still undergoing research. Since by comparison storing CO2 is currently more in demand than these utilization steams, this backgrounder will focus on the former.

Cost Integrity: Capture & Purification

The capture process is by far the most energy-intensive – and difficult to commercialize –part of the value chain, namely due to the nature of the chemical reactions involved in separating the CO2. In absorption, the most common method, a solvent such as monoethanolamide (MEA), or a solvent blend, mixes with and binds to CO2 from flue gas. The CO2-rich solvent stream is heated to an elevated temperature which causes the CO2 to be released and vaporized, while the solvent remains in liquid form. Another way to strip CO2 from the solvent is through pressure swing adsorption (PSA). This involves exposing the solvent to a lower pressure, which causes the dissolved CO2 to separate, which recharges the solvent and creates a high-purity CO2 stream. The liquid solvent regenerates when exposed to a higher pressure, which breaks its bond with the CO2, and the solvent circles back through the loop to be recycled.

Chemical reactions involved in absorption and solvent regeneration require large amounts of energy from reboilers and fired heaters, which can be fueled by a variety of sources. The collection, cooling and pre-treatment of flue gas also demands a significant amount of energy. According to the IEA, most carbon capture processes today consume anywhere from 10% to 40% of the energy generated by the power plant or industrial process. This ratio, called the energy penalty, requires a careful balancing act that can, as much as any single factor, determine the return-on-investment for a given project, and the efficacy of CCS and global climate efforts in general.

The energy penalty, in terms of the cost per ton of captured CO2, depends largely on the capture rate (as determined by the source); the capture method (amine absorption, adsorption, membrane, etc.); the efficiency of the filtration media; the source of energy used to regenerate the filtration media (natural gas boiler, waste heat, coal fired power, renewables); and the concentration of the CO2 in the collected flue gas. The higher the concentration the lower the cost, which according to the IEA, currently ranges from around $15 to $120 per metric ton, depending on the emissions source. The relationship is roughly linear until the carbon concentration falls below around 13-14%, as it does in dilute gas streams such as from power generation plants, at which point the cost increases sharply primarily due to the laws of thermodynamics. Direct air capture costs are significantly higher, between $600 and $1,000 per metric ton, mainly because of the much larger amounts of energy needed to collect and purify carbon dioxide from the atmosphere at its natural concentration of about .04%.

Regulatory factors and the cost of carbon on the open market also affect a project’s energy penalty. In Canada and the EU, every ton released is taxed at a certain rate, so investment is developing fastest where the tax on releasing the CO2 is highest relative to the cost of capturing it. The U.S. is looking to implement an incentive system based on a sliding scale of tax credits to help offset costs for industries that are hard to abate, like cement and steel production. In both cases the goal is to generate the highest concentration CO2 stream possible using the least amount of energy.

Some experts worry that implementing carbon capture technology on a wide scale could increase the consumption of fuels (both fossil and biomass, which has implications on land use) and exacerbate the very problem it is meant to address. This presents a significant obstacle to achieving economies of scale, especially since the higher costs currently associated with capture can make it appear less commercially viable compared to other production methods, such as conventional hydrogen or nuclear energy. Overcoming it requires developing more energy efficient capture technologies and relying on low-carbon energy sources, improving the performance of current capture methods, and using renewable sources to power capture systems while creating regulatory frameworks that help offset the costs associated with them.

Streamlining engineering & process design –

Any significant advance in operational efficiency starts with design. By choosing the right materials, operating conditions, process configurations and equipment, engineers can build a level of ‘cost integrity’ into the system that secures feasible projects and the highest possible return on investment for the life of the project. Any design flaws or inadequate equipment selection at this stage can result in inefficiencies, permanent energy losses, and often additional energy requirements downstream. At the same time, any steps taken to improve heat integration, reduce pressure drop, or enhance heat recovery can significantly lower a facility’s energy, capital and operating costs.

Advanced performance engineering software can determine the economic feasibility of a proposed capture unit years ahead of groundbreaking, making operational and capital expenditure decisions easier to justify and ensuring technical and economic feasibility of the project. Using comprehensive property databanks and algorithms that accurately model the behavior of extremely complex chemical and thermodynamic systems, engineers can better predict reliability issues, identify inefficiencies, estimate energy requirements, assess risk, and address problems before they turn into lost time and money.

It is possible to significantly reduce energy requirements by adjusting the solvent type, concentration, temperature, pressure, and flow conditions to best suit the volume and process. For example, with the right design, amine regeneration applied to coal combustion flue gas can improve from around 5.5 to 3.0 gigajoules per ton of CO2 captured for advanced amines. This translates directly into reduced steam costs and a lower energy penalty. The selection of an efficient solvent or adsorbent can also prevent significant energy losses downstream. Other ways to reduce energy costs include finding the optimal steam supply method, minimizing the inefficiency of the steam extraction at nominal and partial loads, eliminating potential bottlenecks, and recovering waste heat from the capture system for use in the plant’s steam cycle. For example, if the capture system requires high-pressure steam but the existing plant lacks the capacity, adding a steam generator might be necessary.

While engineering models are useful in initial stages, digital twin technology allows engineers to further streamline the design and operation of capture systems by leveraging analytical computer models to improve energy efficiency and reduce costs, both before and after a system is brought online. A digital twin is a software-based virtual replica of an asset or assets, which can simulate the operation of a given system, predict maintenance requirements, and identify areas for improvement. Carbon capture digital twins can simulate the flow of gas and liquid streams through the capture system, including the absorber and desorber units, in high resolution. By adjusting variables such as the temperature, pressure, and flow rate of the streams, engineers can identify the optimal conditions for capturing CO2 with the lowest amount of energy possible. Engineers can also use digital twin technology to monitor the performance of the physical system in real time and compare it to the simulation model, allowing them to quickly detect any deviations or abnormalities that could indicate energy waste.

Digital twin simulation is also indispensable for training. By simulating the operation of the capture system under different conditions and scenarios, personnel gain a better understanding of how the system works and how to optimize it. For example, they can replicate the impact of changes to the temperature, pressure, or flow rate of gas and liquid streams on overall capture efficiency and learn how to adjust these variables to minimize energy consumption without the risk of affecting throughput. High-fidelity operator training systems like these are particularly important for the safe startup of greenfield projects and any new CCS facilities.

Increasing efficiency of heat transfer –

Improving the efficiency and performance of heat generation and transfer equipment throughout the capture process has a direct impact on the cost integrity of any facility, which varies depending on the capacity and chemistry involved. Boilers, pumps, compressors, cooler units and heat exchangers play a vital role in transferring heat between the hot flue gas and the cool solvent or other filtration media, allowing the chemical reactions in the capture process to occur at the right temperatures so that pure CO2 is obtained. After separation, the filtration media is chemically regenerated, usually by heating it again with steam in a reboiler. Plate-and-frame heat exchangers, which consist of thin plates stacked together and separated by gaskets, are typically the most efficient of all designs, frequently reaching efficiency levels of 90%. Shell-and-tube heat exchangers, which consist of a series of tubes inside a larger shell, are also common. Both work by directing the solvent and the flue gas along the correct paths at the optimal flow rates to maximize the transfer of thermal energy.

All heat exchangers are prone to fouling, corrosion, and other maintenance issues that can impact their efficiency, and the acidity of the CO2 and solids in the flue gases exacerbate the problem. Impurities in the flue gas, such as ash and other solid particulates, can build up persistently as fouling on the inner walls of a heat exchanger's tubing, reducing available area for heat transfer, and increasing the need for more energy to regenerate the same amount of filtration media. Also, CO2, when combined in any state with water or other contaminants, is extremely corrosive, as are solvents, especially when mixed with acidic flue gases, all of which can degrade an exchanger’s steel surfaces. A long list of factors and variables, such as the opening temperature of the heat pipe, fluid flow rate, tube angles and spacing, affect the heat transfer efficiency of a heat exchanger. If mismanaged, any combination can easily lead to wasted energy, higher maintenance costs and increased safety risk.

This is why continuously monitoring and managing the performance of heat transfer equipment using a pervasive approach to data collection can be such an effective way to lower the energy penalty of a capture system. If traditional wired instrumentation was ever prohibitive in the past, especially in physically remote settings, wireless technology has proven itself reliable and cost-effective to the point where it now makes sense to deploy high-accuracy sensors at every practical point where it can improve the operator’s heat transfer picture. Even more so given the capability to measure differential pressure, flow, and temperature with high accuracy in real time using a single, self-diagnostic wireless device.

Aggregating more data on heat exchanger performance and steam generation, however, is only valuable if the information is translated into timely, operational decisions that save energy and drive down costs. When integrated with smart field devices and wireless networking infrastructure in a digital plant ecosystem, analytical software applications that are pre-configured for heat transfer equipment make it possible to assign an overall performance rating for each unit indicating process efficiency and asset health. When they begin to move against baseline, the software alerts operators to which assets need attention and why—whether pump cavitation, excessive compressor vibration, or fouling in a heat exchanger tube—before any of them can negatively impact energy costs.

Calculating all those optimal performance baselines is critical, not only for heat exchangers, but also for boilers, pumps, compressors, and other critical equipment, just as precise control is necessary to maintain overall process stability. This is where advanced adaptive process control software can improve the efficiency of a capture system across multiple variables by allowing equipment to run closer to operational and safety constrains. Variable frequency drive motors enable highly precise control of rotating assets, including fans, pumps, and compressors. Energy management information systems (EMIS), meanwhile, can pinpoint root causes of deficient performance in complex heat generation systems. Pre-configured analytical software also exists specifically for optimizing the performance of multi-fuel boilers that use off gases, waste fuels, and biomass to further reduce a system’s carbon footprint while increasing the capacity and stability of steam production.

Balancing capture rate vs. energy use –

If the objective of all carbon capture plants is to isolate as much pure CO2 as possible using the least amount of energy, identifying the ideal balance between capture rate and energy cost is where automation might have the greatest potential to advance the adoption of CCS. In any case, it requires a comprehensive evaluation of several factors and a systematic, multi-dimensional approach. To begin with, rigorous process simulation techniques are critical since they help operators better understand the behavior and performance of their capture system. As one such tool, analytical models use physics and chemistry correlations to accurately predict the interactions between different components, such as in the CO2 absorption or adsorption process and regeneration process, as well as the resulting energy requirements. By simulating different operational scenarios and parameters, it becomes possible to assess the impact of different capture rates on energy consumption, and vice versa, without the risk of real-world experimentation. The techno-economic analysis commonly used to evaluate the viability of a proposed capture system involves estimating the capital and operational costs, including energy consumption, and potential revenue streams associated with the captured CO2 (i.e., from utilization or storage). By modeling scenarios at different conditions, a cost-benefit analysis can determine the optimal balance between capture rates and energy consumption, which helps stakeholders understand the trade-offs and guides decision-making.

Using process simulation to examine different regeneration temperatures, pressures, chemical compositions, and other variables, it is possible to minimize energy requirements while maintaining a satisfactory capture rate. Techniques such as design of experiments, sensitivity analysis and numerical optimization algorithms can find the optimal operating conditions, which may vary depending on specific applications, industrial processes, and geographical locations. Different industries may have different capture requirements or motivations based on their emission profiles and regulatory frameworks. Factors such as the scale of operation, available infrastructure, and policy incentives can also influence the optimal balance. Conducting site-specific evaluations and considering industry-specific factors help in tailoring the capture system to meet the specific needs and objectives of each case.

Integrated control systems play a crucial role in managing capture rates and energy consumption in carbon capture systems by continuously monitoring operating conditions, performance metrics, and relevant parameters, such as temperature, pressure, flow rate and composition. By continuously analyzing the incoming flue gas composition, flow rate and other relevant factors, the control system can optimize the capture process in real time. It can also regulate parameters such as adsorbent bed temperature, adsorption pressure, or solvent flow rate to ensure the CO2 capture rate remains within the desired range. This type of control scheme helps maintain a high capture efficiency while minimizing energy consumption and optimizing system performance. Real-time optimization ensures that the energy-intensive steps of the process occur under the most efficient conditions possible.

In many cases carbon capture systems are integrated with existing processes, such as power generation or industrial production. Advanced process control (APC) software can coordinate the operation of the carbon capture system with the rest of the facility, optimizing energy usage and load distribution, allowing for better use of resources, and maximizing the overall efficiency of the system. The value of APC extends to new facilities as well, particularly within the adsorber/absorber unit, due to the complex multivariable process requirements. Such integrated control systems leverage advanced machine learning algorithms to optimize performance. By analyzing historical data and patterns, they help operators identify new opportunities for efficiency, develop better predictive models, and recommend operational adjustments. This kind of intensive, data-driven optimization approach helps to continuously improve both capture rates and energy consumption by adapting to changing conditions, which improves the economic viability of carbon capture systems over the long term.

Density metering and gas analysis are also important for maintaining the ideal capture-to-energy balance. By monitoring the chemical composition of complex fluids, density meters provide crucial information about the concentration of capturing agents, which plays a vital role in controlling solvent regeneration and allowing precise timing across the process to optimize energy consumption. Density metering instantly detects any deviations in composition, enabling prompt corrective actions, contributing to troubleshooting, throughput and safety. Continuous gas analyzers, types of which include infrared, electrochemical, cascade laser, and gas chromatograph, provide real-time data on chemical composition and CO2 concentration. This enables the control system to regulate the amount of heat used for solvent regeneration. Gas analyzers also aid in evaluating overall system performance and simplifying the regulatory reporting process, which help operators make better operational decisions and achieve the desired capture-energy balance.

Equipment Integrity: Compression/Liquefaction

After the purification process, the next step usually involves compressing gaseous CO2 to the appropriate pressure and temperature needed to liquefy it. Operators might choose to transport CO2 as a liquid to enhance efficiency through increased density, although water removal is necessary to prevent freezing and corrosion. Others will opt to ship it as a gas, using existing infrastructure and reducing energy requirements for refrigeration and compression but dealing with phase changes and pressure drops. As mentioned above, carbon dioxide enters a supercritical state at temperatures above 87.8 degrees F (31.0 degrees C) and pressures above 73.8 bar, exhibiting properties of both gases and liquids. The supercritical state allows for higher flow rates and is often more cost-effective for longer distances due to lower viscosity compared to liquefied CO2, but proper pressure and temperature control must remain constant along the transport route.

Carbon dioxide condenses as it moves through several stages of compressors and intercoolers arranged in a so-called compressor train, a process that requires careful attention to measurement and control, especially given the complex properties of high-density CO2. Rotating equipment, namely pumps and compressors, which runs continuously at high RPMs, day-and-night, is vulnerable to wear and tear from mechanical stress, leading to potential breakdowns. This is serious cause for concern since if just one compressor in the train fails then the entire process, and the whole capture plant, can easily follow.

Another top priority during compression is to prevent leaks. Containment loss can become a major problem when dealing with any process fluid, both in terms of lost production and environmental and safety concerns. This is especially true when the ‘product’ is a highly concentrated, potentially corrosive, lethal asphyxiant. Keeping all the CO2 in is just as important as keeping contaminants such as sulfur and water out. Operators carefully track purity on molecular level throughout the system to meet safety standards, prevent downstream contamination, control corrosion, and facilitate fiscal reporting.

While initial compression usually takes place at the same location as the capture system (onsite at a power plant, for example) prior to entering a pipeline or bunkering on a vessel for transport, pressurizing the CO2 further may be necessary before injecting it into the ground. This increases storage capacity, improves injectivity by facilitating penetration deep into rock formations, ensures better reservoir conformance by filling pore spaces, and reduces mobility and leakage risks.

Improving equipment availability –

It is possible to significantly improve the performance and reliability of critical equipment in the compression stage by focusing on three capabilities: first, accurate measurement of complex process variables and precise control of pumps, compressors, intercoolers, heat exchangers and other equipment; second, implementing preventive maintenance and automating maintenance rounds; and third, mitigating the effects of corrosion on equipment, vessels, pipes, and valves.

Like the capture process, aggregating data across the compression system with wireless self-diagnostic instrumentation gives operators the greatest visibility of asset health and performance. Advanced pressure sensors provide the accuracy and resolution needed to keep CO2 at the correct density, while temperature transmitters use thermocouples, resistance temperature detectors (RTDs) or non-invasive sensing technology to accurately measure fluid temperature at key stages throughout the process. Flow meters employ various principles such as thermal, differential pressure or ultrasonic methods to calculate fluid volume and mass flow rates, and to monitor the characteristics of CO2 as it undergoes compression. Combined with advanced control algorithms, these instruments make it possible to adjust and optimize the compression process in real time, maximizing capacity and preventing shutdowns. They also enable operators to keep the CO2 in a constant liquid or supercritical state and ensure it remains condensed within the specified parameters for efficient and safe transportation.

Digital, anti-surge safety valves are another valuable tool for ensuring compressor reliability. ‘Surge’ refers to the operating point at which a centrifugal compressor reaches its peak head capability and minimum flow limits. When the pressure behind the compressor is higher than the compressor outlet pressure, the fluid tends to reverse. As a result, the compressor loses the ability to maintain peak differential pressure when a surge occurs, and the entire system becomes unstable. Surging can also cause the compressor to overheat to the point of causing damage to bearings and rotors. Digital anti-surge valves detect when a process compression stage is at risk of surging and automatically increases flow through the compressor, resulting in stable working conditions and more efficient performance.

Conducting preventive maintenance –

Preventive, condition-based maintenance technology involves the use of sensors and predictive data algorithms to monitor equipment performance and detect potential issues before they cause major problems. By addressing issues early on, maintenance personnel can perform repairs or replacements as needed, reducing the likelihood of unplanned downtime, and minimizing the risk of equipment failure. This also helps to optimize equipment performance, reducing the overall energy consumption and cost of operation. By monitoring key performance metrics and adjusting set points as needed, operators can ensure that critical equipment throughout the compressor train consistently operates at peak efficiency, reducing wear and tear on components, and extending the life of the equipment.

Instead of following fixed maintenance schedules, preventive maintenance technology such as asset management software, edge computing devices, self-diagnostic valves, and remote monitoring solutions, enables a more efficient, predictive approach to conducting inspections and repairs. By quickly analyzing equipment data and considering factors such as operating conditions, load demands, and asset health, preventive software tools schedule maintenance based on the actual condition and performance of the equipment, which optimizes maintenance efforts, reduces unnecessary maintenance tasks, and focuses resources on critical areas that require attention. Thanks to advances in secure data networking, capture plant managers are increasingly able to rely on offsite monitoring, testing, and troubleshooting expertise made available through remote maintenance services that are customizable according to each facility’s unique operational needs.

Controlling corrosion –

Pure CO2 is corrosive due to its ability to react with moisture, forming carbonic acid, which gradually breaks down protective oxide layers on metal surfaces, leading to corrosion and material degradation. Factors such as temperature, pressure, and concentration influence the corrosive potential of CO2. Materials vary in their susceptibility to corrosion, necessitating careful material selection and corrosion-resistant measures in all systems and equipment that can come into contact with CO2. This is why managing corrosion throughout the compression/liquefaction stage (and the rest of CCS value chain) using advanced automation technology can have such a significant impact on process efficiency, environmental containment, and cost effectiveness.

Digital solutions, like wireless non-invasive transmitters and linear corrosion probes, help ensure equipment integrity, preventing failures, leaks, or ruptures that could compromise safety and environmental protection. Corrosion monitoring software provides insight into asset health and alerts operators to flow restrictions and pressure drops, promoting smoother CO2 flow, reducing energy consumption, and enhancing overall compression system efficiency. Purpose-built final control elements, including butterfly and ball valves rated at pressures up to 100 bar, reduce health, safety, and environmental risk by preventing emissions from corrosion-induced leaks or failures. Hot-gas bypass valves also help improve cost effectiveness by avoiding compressor over-pressure events and reducing overall maintenance requirements, unplanned downtime, and the need for repairs or replacements.

Product Integrity: Transportation

Transportation involves three main operational objectives: first, keeping the compressed CO2 pure and safely contained, whether in a pipeline or bunkered aboard a ship; second, maintaining it at the ideal pressure and temperature; and third, correctly measuring and documenting exact quantities of CO2 as they move from the point of capture to the storage site. Companies and government agencies are working together to develop new methods and technologies to meet these challenges since as CCS infrastructure scales up, so does the need to standardize procedures and support commercial invoicing and fiscal reporting.

Compressed CO2, whether in its liquid or supercritical phase, tends to leak through joints and other fittings, which means all equipment along the transport route must be carefully designed and maintained with appropriate seals, gaskets, and welds. Valves and other final control elements are potential leak points, especially in high-pressure situations, as are any places where probes penetrate pipe walls. Temperature and pressure can fluctuate during transport due to changes in ambient conditions or shipping operations, which if not properly monitored and controlled, can result in undesired changes in the carbon dioxide’s physical state, such as solidification or sublimation. This often leads to blockages, clogs, or inefficiencies in a pipeline system, restricting flow, reducing capacity, and eventually causing equipment failure and loss of containment. Improper pressure control, meanwhile, can lead to over-pressurization or under-pressurization, which can compromise the ‘product integrity’ of the CO2, as well as the storage vessels, pipelines, or containment systems. Corrosion is always a predominant concern, not only because of what can happen in the case of a ruptured pipe or tank, but also because of how letting in contaminants like water and hydrogen sulfide dilutes and chemically alters the CO2, leading to more corrosion and problems downstream. The fact is that small leaks are inevitable; an operator’s mandate is to detect and deal with them before they develop into bigger problems. Supervisory control and data acquisition (SCADA) systems for pipeline transportation applications can aid operators in these tasks.

A pipeline carrying supercritical CO2 can be more prone than liquid hydrocarbon or natural gas pipelines to running ductile fractures, which can destroy long sections of line because of smaller ruptures. When released from a pipeline, CO2 is heavier than air and high-rate releases will form clouds of cold dense gas fog. Upon warming, CO2 plumes flow considerable distances from the pipeline unobserved, traveling over terrain, displacing oxygen while settling or filling in low areas. Oxygen displacement by CO2 gas can cause asphyxiation and lead to death. It also starves any equipment that burns fuel, causing it to shut off. Moving liquified CO2 in pipelines usually requires specialized cooling equipment and additional energy resources to maintain temperatures slightly below ambient levels, but above the point where steel becomes brittle and begins to weaken. However, the lower temperature and pressure needed for liquified CO2 compared to supercritical fluids or gases does reduce the relative potential for pipeline ruptures.

The formal exchange of ownership of carbon dioxide from the capturing entity to the transportation and storage provider is called custody transfer. The technologies that support custody transfer play a crucial role in the market mechanics of reporting for carbon tax credits and invoicing for storage services. Precise measurement and reporting of the quantity and quality of the transferred CO2 are essential, including detailed documentation of volume, concentration, pressure, and temperature, which serve as the basis for calculating the exact carbon emissions reductions achieved. Receiving entities use the custody transfer data trail to track the volume of CO2 stored and calculate how much to invoice the capturing entity. Operators use high-resolution measurement systems to ensure accurate and transparent financial transactions, which is necessary to meet regulatory standards and facilitate carbon commerce at scale.

Managing purity, density & containment –

Advanced automation tools can be indispensable for mitigating leaks, contaminants, temperature excursions, and other upsets before they alter the physical characteristics of transported CO2 or cause infrastructure failures. Coriolis flow meters, for example, are ideal for measuring the precise volume and characteristics of moving fluid. The mass balance of compressed CO2 is calculated by analyzing the flow data with simulation software platforms that use transient thermohydraulic models based on pressure wave propagation to predict, detect and localize potential leaks in real time.

When deployed in mass on long stretches of remote pipeline, advanced wireless temperature, pressure, and corrosion sensors provide the clearest possible picture of any changes that could affect the density of CO2 in transport, giving operators the time and information required to head off major disruptions. As critical as final control elements are to ensuring stable pipeline operation—and as prone as they can be to leaks—regulatory standards specify certain control and safety valve designs, usually with special packing and redundant seals, rated for the range of pressures and corrosive conditions encountered when shipping CO2. Without them it can be difficult to prevent issues like soft-seating and seal deterioration or control the risk of CO2 sublimation due to contamination or pressure loss.

Facilitating custody transfer –

Flow computers are integral to the smooth operation of custody transfer and fiscal metering in the context of capturing and storing carbon dioxide. These devices play a vital role in accurately calculating flow rates, volumes, and other essential parameters required for precise measurement in high-pressure and high-flow scenarios. Flow computers integrate with the measurement and control systems at the capture and storage sites, working alongside Coriolis flow meters, pressure sensors and temperature gauges. They receive instantaneous data from these instruments and employ advanced algorithms and mathematical models to calculate the exact flow rate and volume of CO2 in transit.

A key feature of flow computers is their ability to manage a wide variety of measurement units, conversions, and corrections, which goes a long way in obtaining consistent, accurate results. On-board software accounts for variables such as temperature, pressure, density, and compressibility of the CO2, thus providing tailored and precise measurements specific to custody transfer conditions. Beyond measurement calculations, flow computers offer additional functionalities such as data logging, data communication and remote reporting capabilities. Advanced flow computers can store measurement data for auditing purposes and historical analysis, generate reports to comply with regulatory requirements, support invoicing processes, and enable seamless data integration with higher-level control systems or enterprise-level software.

Electric valve actuators are another critical link in the chain since they automate the correct positioning of control elements during custody transfer. This precisely regulates flow rates and pressures to ensure they are consistent with all required specifications. Importantly, electric actuators can calculate any lost CO2 by tracking the exact amount of fluid that enters and exits the custody transfer system. They also ensure that operators receive the maximum amount of carbon tax credits available and let storage service providers accurately invoice customers. Using electric valve actuators in custody transfer applications also helps to reduce operating costs since they typically require less maintenance and are more energy-efficient than traditional hydraulic actuators.

Cascade laser gas analyzers, which use a laser beam to measure the absorption of light by different gases, are a recent innovation that offer key benefits in custody transfer applications. Primarily they detect chemical compounds, such as nitrogen, oxygen and other gases that can affect the quality and suitability of CO2 for storage, which helps operators prevent contamination and other serious problems that could multiply downstream. Enhanced continuous analyzer designs allow for the measurement of up to five components simultaneously, down to ultra-low concentration levels, with the lowest possible sensitivity drift in response to temperature and density changes. Gas analyzers like these are also able to calculate exact amounts of any CO2 lost in transit, ensuring sufficiently accurate measurement of fluid quantities for fiscal reporting, invoicing and regulatory compliance purposes.

Emerson recently began offering a turnkey custody transfer solution in the form of a compact, standalone skid with all necessary sensors, valves, actuators, filters, pipes, flow computers, analyzers, controllers and wiring pre-installed. Operators can commission these skids at points between the capture and storage sites, or any place where they need precise, repeatable continuous monitoring and control of the metering process to simplify regulatory compliance and minimize risk.

Performance Integrity: Injection & Geologic Storage

Geologic storage is unique in how it involves many basic oil and gas exploration and production principles, except in reverse. Instead of finding a subsurface formation and pumping every hydrocarbon molecule out of it, the goal is to inject as much CO2 as possible down the well up to the point where the geology remains sound, and then keep it there under pressure indefinitely. Many of the same kinds of efficiency, reliability, and safety challenges apply to standard onshore exploration and production (E&P) operations, with similarities to natural gas fracking, but there are other special considerations due to the novelty, at least in practice, of pumping pressurized liquid greenhouse gas deep down into the earth on a large scale and keeping it there indefinitely.

Three main types of geologic formations are suitable for long-term geologic storage, depending on their properties and characteristics: aquifers, abandoned wells, and unmineable coal seams. Vast underground formations filled with brackish water called deep saline aquifers several kilometers below the surface and have the capacity to store significant volumes of liquified CO2 without encroaching on freshwater supplies. Saline aquifers exist globally, as do geologic reservoirs that oil and gas companies have fully exploited for hydrocarbon extraction, many of which can serve as suitable storage sites. These reservoirs have well-established geologic properties and infrastructure, making them particularly attractive for CCS use. Operators can also inject CO2 into unmineable coal seams, particularly those with deep underground deposits where the coal absorbs it, more effectively trapping the pressurized fluid underground. A suitable number of coal formations are known to exist in North America, Europe, Asia, and Australia.

For operators of these sites, the physical security of the formation and the possibility of CO2 leakage, which could occur through faults, fractures, or improperly sealed wells, is a chief concern. Leakage at the storage site can cause serious environmental damage, pose safety risks, lead to fines, lawsuits and permitting issues. An additional challenge is the potential for induced seismicity, or earthquakes triggered by the injection of large volumes of CO2 into the subsurface. Operators must carefully monitor injection rates, pressures, and volumes to avoid exceeding the capacity of the formation and inducing seismic events. Careful monitoring, control, and maintenance of the well, injection equipment, and the formation itself are essential to make sure the stored CO2 remains permanently contained and does not migrate out of the storage formation over time.

Assessing geologic characterization –

Much of the critical work in the injection and geologic storage process happens long before any carbon pumps downhole. Securing a government permit to use a reservoir or formation for long-term storage requires substantial research and investment. Private operators are responsible for proving that the proposed storage site will be able to safely sequester thousands of cubic tons of liquified CO2 under pressure, often as high as 10,000 psi or greater, for a period of at least one hundred years.

Obtaining and accurately assessing a complete characterization of the reservoir requires specialized downhole gauges to accurately measure pressure and temperature. Subsurface science and engineering software analyzes the data with seismic modeling technology to detect the performance integrity of the well, mineral properties, rock and fluid distribution, and other key factors. Using subsurface engineering software helps to quickly evaluate and assess candidate storage sites while reducing the time needed to gather information to support permitting, which translates into broader CSS infrastructure growth. Operators often report the findings from the same analysis in measurement, monitoring and verification (MMV) plans, which all geologic storage sites must file under regulation. Crucially, by determining the best well design and injection rate, subsurface engineering software also helps to give stakeholders enough confidence to break ground on new CCS projects and rest assured they will stand the test of time.

Monitoring and controlling injection –

In addition to streamlining the site exploration and drilling process, operators use subsurface engineering software after CO2 injection begins. To accurately calculate, monitor, and control the rate of CO2 flowing underground, specialized software simulates the injection process by creating accurate models of the formation and the associated wellbore infrastructure. These models incorporate enormous amounts of complex data about reservoir properties and fluid dynamics to help operators assess the impact of different injection scenarios, optimize injection rates and pressures, and predict the behavior of the subsurface CO2 plume over time. This includes predicting risks such as leakage pathways, pressure buildup, and induced seismicity, allowing operators to take appropriate mitigation measures.

Subsurface engineering software integrates with pressure, flow and temperature instrumentation above and below ground to provide continuous real-time feedback on the site’s performance and detect deviations from expected well behavior. This critical information is translated into operational work by advanced process control systems designed to provide precise, reliable automated control of pumps, compressors, valves, and other critical equipment calibrated specifically for the storage site’s unique parameters. APC systems alert operators to potential upsets in the injection process, seamlessly integrating with simulation and engineering software to continuously provide the clearest possible assessment of the situation underground.

Specialized hardware, including cryogenic valves rated to withstand large temperature and pressure excursions, enables precise control over the injection of liquified or supercritical CO2, improving efficiency and uptime while preventing leaks and safety incidents. Seismic analysis software compares the data gathered from these sensors and other instrumentation with the analysis done before injection begins to validate that the CO2 remains safely underground, and that the storage site remains unaffected by any activities related to drilling, injection, and well operation.

As with transportation, operators of permanent storage sites also oversee custody transfer and fiscal metering duties to facilitate proper billing and document the exact volume of CO2 received and sequestered in the storage formation. Custody transfer technology also aids in regulatory reporting and safety compliance purposes. Automated inventory management is another beneficial technique to deploy wherever temporary buffer tanks hold CO2 above ground before injection. Tank gauging hardware, such as guided wave radar level transmitters, works hand-in-glove with pre-configured software to calculate tank and CO2 mass volumes and track fluid movement in real time, preventing overfills and improving injection efficiency.

Conclusion: From ‘Boondoggle’ to Net-Zero Imperative

Today the public perception surrounding CCS is undergoing a remarkable transition. Once regarded skeptically as a wasteful endeavor, CCS technology has emerged as an indispensable tool to help mitigate the devastating impacts of climate change. The pressing need to curb greenhouse gas emissions and combat global warming has reshaped the narrative around CCS, elevating it to a position of strategic significance. This escalating urgency to transition to a low-carbon economy has begun to underscore the viability of CCS as a direct, practical pathway to achieving deep decarbonization in sectors that face challenges in electrification or rely heavily on fossil fuels.

The growing recognition of CCS as an essential climate change tool is evident in the mounting political will, international collaborations, and investments in research, development, and deployment. Governments, industries, and organizations across the globe are actively pursuing ambitious projects and passing landmark legislation to push CCS, acknowledging its potential to meet ambitious climate targets and accelerate the shift towards sustainability. Nevertheless, significant obstacles and hurdles persist on the journey to widespread CCS adoption, including the need to foster further technological advancements, cost optimization, public acceptance, and regulatory frameworks that support implementation. Addressing these challenges necessitates continuous innovation, collaboration, and even more robust policy support to realize large-scale deployment of CCS.

As the New York Times recently said about carbon capture, “once considered by many to be a boondoggle, the technology has matured.” This reflects the quickly evolving understanding of the potential and significance of CCS that few other emissions mitigation methods can claim. Given the urgency behind decarbonization and the crossroads at which the world now stands, embracing CCS as a critical component of a comprehensive toolkit to combat climate change has become an imperative. There is simply too much to lose. Fortunately, with the advances made possible by innovative automation technologies and continued public and private collaboration, CCS is set to play a pivotal role in protecting the environment for generations to come.